NI-209 TEMPERATURE MODEL ILLUSTRATE  BOOK

EXPLAIN AS FOLLOWS：

- DISPLAY TEMPERATURE (NORMAL ATMOSPHERIC TEMP.)

- SET BY "SET" KEY.

- SELECT OF UP OR DOWN.

- "    " DISPLY  FOR  ZERO UNDER.

- "OUT" TO CONNECT O/P SIGNAL.

FUNCTION ILLUSTRATE：

- PERMIT SET TEMP. RANGE ﹢40℃ TO ﹣40℃﹒

- DIFF TEMP. SET RANGE 1℃ TO 9℃﹒

- POWER 110/220VAC

- SELECT OF LOCK FUNCTION OR CUT-OFF TO DELAY 3 MINUTES﹒

- DUE TEMP.SENSOR BROKEN STRINGS OR UNUSUAL TO DISPLAY "E1.99"﹒

- RELAY ACTION：TEMP. IS NOW ＞ CONSTANT TEMP. + RELAY ON TEMP﹒

TEMP. IS NOW ＜ CONSTANT TEMP. VALUE THAN RELAY OFF﹒

OPERATION ILLUSTRATE：

YOU THINKING CONTROLER TEMP.RANGE 10℃ TO 13℃ , DIFF TEMP 3℃ ﹒

1. "SET" KEY PRESS ABOUT 1 SECONDS. PLEASE.ADJUSTMENT       TO 10℃. ONCE MORE PRESS "SET" KEY GO TO NORMAL ATMOSPHERIC TEMP.

2. DIFF TEMP. PRESS "SET" KEY ABOUT 3 SECONDS TO OPEN, 1 DIGITAL IS DOING BRI ADJUSTMENT        KEY TO 3﹒

3.PRESS "SET" KEY ABOUT 5 SECONDS OPEN FOR CUT-OFF POWER TO DELAY OR LOCK FU, WAIT FOR L0 TO L3 SYMBOL TO APPEAR﹒

CUT-OFF POWER TO DELAY AS FOLLOWS：

| HAVE CUT-OFF POWER DELAY | NO HAVE CUT-OFF POWER DELAY |
| --- | --- |
| L0：UNLOCK | L2：UNLOCK |
| L1：ALL LOCK | L3：ALL LOCK |

ASSEMBLE DIAGRAM：

CONNECT POING COMTROLER COMPRESSOR ELECTROMAGNETIC SWITCH